Section 1 - Chemical Product and Company Identification

Product/Chemical Name:  Alumina Trihydrate

Synonyms: Alumina Hydrate, ATH, Aluminum Hydroxide, Trihydrate D’aluminae, Hydrate d’aluminae, Gibbsite, Hydroxide d’aluminum, Alumine hydrate, hydrated alumina, Trioxyde d’aluminum, Aluminum Trihydroxide, Al203·3H2O

General Use: refractory material, abrasive, fire retardant and smoke suppressant for plastics, ceramic material.

Manufacturer:  The R. J. Marshall Company			Emergency Phone:  (800) 424-9300

26776 W. 12 Mile Road				Date Revised:  2/7/14

Southfield, MI  48034-7807			Preparer: Stephanie Nichols

Phone (248) 353-4100, Fax (248) 948-6460

| Section 2 - Hazards Identification HMIS |  |
| --- | --- |
| H F R | 1 0 0 |
| PPE† E †Sec. 8 |  |

Hazard Pictogram:  None

Hazard Statements:  None

Precautionary Statements:

P261:  Avoid breathing dust.

P280:  Wear eye protection.

P285:  In case of inadequate ventilation wear respiratory protection.

P301+313+330:  IF SWALLOWED:  Rinse mouth, get medical advice.

P305+351+338:  IF IN EYES:  Rinse continuously with water for several minutes.  Remove contact lenses if present and easy to do-continue rinsing.

P302+352:  IF ON SKIN:  Wash with soap and water.

P402:  Store in a dry place.

Section 3 - Composition / Information on Ingredients

| Ingredient Name |  | CAS Number |  | Weight |
| --- | --- | --- | --- | --- |
| Alumina Trihydrate | 21645-51-2 |  | 100% |  |

Section 4 - First Aid Measures

Inhalation: If overcome by high dust concentrations, remove to a ventilated area.

Eye Contact:  Flush eyes thoroughly for several minutes taking care to rinse under eyelids.  Do not scrub.  Abrasion may cause irritation.  If discomfort continues, continue to wash with water.  If irritation persists, consult a physician.

Skin Contact: Wash skin thoroughly with soap and water.  Consult a physician if irritation persists.

Ingestion: If swallowed, dilute with large amounts of water.  Seek medical advice.

Most important symptomes and effects, both acute and delayed:  None

Indication of any immediate medical attention and special treatment needed:  None

Section 5 - Fire-Fighting Measures

Flammability Classification: Non-combustible

Extinguishing Media: Use extinguishing measures appropriate to the surrounding fire.

Unusual Fire or Explosion Hazards: None known.

Hazardous Combustion Products: None.

Fire-Fighting Instructions: None

Section 6 - Accidental Release Measures

Personal precautions, protective equipment, and emergency procedures:  Avoid dust formation.  In case of inadequate ventilation wear respiratory protection.

Methods and materials for containment and cleaning up:  Sweep up and dispose in accordance with local regulations.

Regulatory Requirements: Follow applicable OSHA regulations (29 CFR 1910.120).

Section 7 - Handling and Storage

Handling Precautions: Avoid generating dust during handling.

Storage Requirements: Keep material dry.

S-phrases:  S8, S36/37/39

Section 8 - Exposure Controls / Personal Protection

Engineering Controls:

Ventilation: Provide general or local exhaust ventilation systems to maintain airborne concentrations below OSHA PELs (Sec. 2). Local exhaust ventilation is preferred because it prevents contaminant dispersion into the work area by controlling it at its source.

Administrative Controls:

Protective Clothing/Equipment: Wear tightly fitting safety goggles or safety glasses with side shields.

Contaminated Equipment: Separate contaminated work clothes from street clothes. Launder before reuse. Remove this material from your shoes and clean personal protective equipment.

Comments: Never eat, drink, or smoke in work areas. Practice good personal hygiene after using this material, especially before eating, drinking, smoking, using the toilet, or applying cosmetics.

|  | OSHA PEL |  | ACGIH TLV |  |
| --- | --- | --- | --- | --- |
| Ingredient | TWA | STEL | TWA | STEL |
| Alumina Trihydrate | none estab. | none estab. | none estab. | none estab. |
| Nuisance Dust | 15 mg/m3 total, 5 mg/m3 respirable | none estab. | none estab. | none estab. |

Section 9 - Physical and Chemical Properties

| Physical State: white powder Appearance and Odor: white odorless powder Odor Threshold: n/e Vapor Pressure: n/e Vapor Density (Air=1): n/a Formula Weight: n/a Density: 8-80lb/ft3 Specific Gravity (H2O=1, at 4 °C): 2.42 pH: 8.5-10.2 (20% solution) Flash Point: Not applicable. Flash Point Method: Not flammable. Burning Rate: Does not burn. Auto-ignition Temperature: Does not ignite. Explosive properties: Non-explosive. | Water Solubility: Insoluble Other Solubilities: Soluble in alkaline aqueous solutions or in HCL, H2SO4, and other strong acids in the presence of some water. Boiling Point: n/a Freezing/Melting Point: 3700°F (2038°C) Viscosity: n/a Refractive Index: n/a Surface Tension: n/a % Volatile: n/a Evaporation Rate: n/a Thermal Decomposition: >200C Octanol-Water coefficient: None |
| --- | --- |

Section 10 - Stability and Reactivity

Stability: Alumina trihydrate is stable at room temperature in closed containers under normal storage and handling conditions.

Polymerization: Hazardous polymerization cannot occur.

Chemical Incompatibilities: Strong acids and bases.  Alumina trihydrate reacts vigorously with strong acids and will dissolve in caustic solutions.

Reactivity with Heat: When exposed to fire or heat, hydrated alumina loses its water of crystallization beginning at 392F (200C).

Hazardous Decomposition Products: n/a

Section 11- Toxicological Information

Toxicity Data:*

| Primary Entry Routes: Inhalation Inhalation: Inhalation of high concentrations of this inert nuisance particulate can result in mild irritation of the respiratory tract. Eye: May cause irritation through mechanical abrasion. Skin: May cause irritation through mechanical abrasion. Ingestion: Unlikely. | Acute Oral Effects: None known. Chronic Effects: None known. Carcinogenicity: Neither this product nor any of its components are considered carcinogenic by OSHA, IARC, NTP, or ACGIH. |
| --- | --- |

Section 12 - Ecological Information

Fish toxicity:  LC50>100 mg/l in Salmo trutta.

Invertebrate toxicity:  EC50>100 mg/l in Daphnia Magna

Algae toxicity:  EC50 >100 mg/l in Selenastrum capricornutum

Persistance and degradability:  Not applicable for inorganic substances.

Bioaccumulative potential:  Not applicable

Mobility in soil:  Not applicable.

Section 13 - Disposal Considerations

Disposal: Recycle if possible or landfill.  This substance is inert and does not require special disposal methods.  Follow applicable Federal, state, and local regulations.

Section 14 - Transport Information

DOT Transportation Data (49 CFR 172.101):  This product is not classified as dangerous under the transport regulations for road, rail, sea, or air transport.

Section 15 - Regulatory Information

EPA Regulations:

RCRA Hazardous Waste Number: Not listed (40 CFR 261.33)

RCRA Hazardous Waste Classification (40 CFR 261.??): Not classified

CERCLA Hazardous Substance (40 CFR 302.4) not listed

SARA Toxic Chemical (40 CFR 372.65): Not listed

SARA EHS (Extremely Hazardous Substance) (40 CFR 355): Not listed

OSHA Regulations:

Air Contaminant (29 CFR 1910.1000, Table Z-1, Z-1-A): Not listed

OSHA Specifically Regulated Substance (29CFR 1910.????)

Inventory Lists:  This product is listed on the following inventory lists:

| TSCA | Australia AICS | Canada DSL | China IECSC | Europe EINECS | Japan ENCS | Korea ECL |
| --- | --- | --- | --- | --- | --- | --- |
| Listed | Listed | Listed | Listed | 244-492-7 | 1-17 | KE-00980 |

| New Zealand NZIoC | Swiss Giftliste | Philippines PICCS | Taiwan NECI |
| --- | --- | --- | --- |
| Listed | G-4621 | Listed | Listed |

International Regulations:

Regulation (EC) No 2037/2000 (Ozone Depleting Substances); Not applicable

Regulation (EC) No 850/2004 (Persistant Organic Pollutants):  Not applicable

Regulation (EC) No 689/2008 (Export and Import of Dangerous Substances):  Not applicable

Directive 2002/95/EC (RoHS):  Not applicable

Directive 2002/96/EC (WEEE):  Not applicable

Directive 1999/13/EC (VOC):  Not applicable

Restirctions according to TITLE VIII of the Regulation (EC) No 1907/2006 (REACH):  None

S-phrases:

S8:  Keep container dry.

S36/37/39:  Wear suitable protective clothing, gloves, and eye protection.

WEEE:  Waste Electrical and Electronic Equipment

VOC:  Volatile Organic Compounds

RoHS:  Restirction of Hazardous Substances

Section 16 - Other Information

Prepared By: Stephanie Nichols

Revision Notes: Updated to GHS

Product Grades Available from the R. J. Marshall Company (this list may be incomplete):

| DF40 | OFI | A202 (A,W) | AH170(A,C,V) | DGXW1 |
| --- | --- | --- | --- | --- |
| DF45 | OFIII | A203H | AH190 | DGXW2 |
| DF74 | OFV | A204(A,W) | AH255(A,W) | EM107 |
| DF80 | OFVS | A205(A) | AH270(A,C,V, W) | EXT DF 168 |
| DF80S | OFVI | A206(A,H,W) | AH280(A,C,V,W) | ATH-A |
| DF85 | OF7 | A208(A) | AH285W | ATH-B |
| DF111 | OF136 | A210(A) | AH290(A,C,V,W) | DT1080 |
| DF119 | OF250 | A210SP | AH290S | KX-1 |
| DF121 | OF255A | A212(A) | AH298(A,C) | DT1186 |
| DF132 | OF300 | A245(W) | AH330(A,C,V,W) | STM1410 |
| DF161 | OF500 | A236HL | AH331(A) | AC470AN |
| DF174 | OF505 | A200SB1 | AH430(A,C,H,V,W) | AC480AN |
| DF211(A) | OF600 | RC802(A) | AH610 |  |
| DF221 |  |  | AH690 |  |
| DF225 | OR150 | A102 | AH171(A) |  |
| DF230 | OR250 | A104 | AH271(A) |  |
| DF240 | OR255A | A105 | AH281(A) |  |
| DF270 | RC822 | A106 | AH291(A) |  |
| DF315 | A4555 | A108 | AH331(A) |  |
| DF320A | 8515 | A110 | AH431(A) |  |
| DF325(C,W) | HF136 | A112 | AH691 |  |
| DF420 |  | A130 | AF330(A) |  |
| DF1045 | MX100 | A145 |  |  |
| DF1051 | MX104 | A104A |  |  |
| DF2040 | MX200 |  |  |  |
| DFG |  |  |  |  |

Note:  This includes all EXP ATH blends.

Disclaimer: Information contained herein is presented in good faith and is based on data believed to be accurate.  However no warranty is expressed or implied regarding this information or the results obtained from the use of this Material Safety Data Sheet, whether it originates with The R. J. Marshall Company or others.  This Safety Data Sheet relates only to the specific material designated herein.  It does not relate to use with other material or processes.  This information is supplied with the condition that the user will make appropriate determination as to its suitability for their purpose prior to using it.